# DTN VDES Business Simulator

Store-Carry-Forward satellite network business modelling for maritime VDES/AIS services

## Table of Contents

- Overview

- Architecture

- DTN Contact Plan Model

- Business Model (ARPU)

- Cost Structure

- Financial Projections

- KPIs & Outputs

- Parameter Reference

- Default Scenario

- Key Formulas

- QoS Enforcement Logic

- Infrastructure Scaling Modes

- Known Limitations

- Glossary

## Overview

The DTN VDES Business Simulator is a single-page interactive tool (index.html) that combines:

- A DTN (Delay-Tolerant Networking) contact-plan capacity model for a LEO satellite constellation serving maritime vessels via VHF/VDES

- A freemium ARPU (Average Revenue Per Unit) stack based on IMO e-Navigation services

- A 10-year cashflow, P&L, and ROI projection with infrastructure scaling modes

It is designed for telecom engineers and business analysts evaluating the commercial viability of satellite VDES operators — specifically nanosat/smallsat constellations targeting the maritime industry.

## Architecture

┌──────────────────────────────────────────────────────────────────────────┐
│                        DTN VDES Business Simulator                       │
│                                                                          │
│  ┌─────────────────┐   ┌────────────────┐   ┌──────────────────────┐   │
│  │  DTN Contact    │   │  Business Model│   │  CAPEX / OPEX        │   │
│  │  Plan Engine    │ → │  (ARPU tiers)  │ → │  Cost Structure      │   │
│  │  (Capacity)     │   │                │   │                      │   │
│  └─────────────────┘   └────────────────┘   └──────────────────────┘   │
│           │                     │                       │               │
│           └─────────────────────┴───────────────────────┘               │
│                                 │                                        │
│                    ┌────────────▼───────────┐                           │
│                    │  Financial Projections │                           │
│                    │  Cashflow / P&L / ROI  │                           │
│                    └────────────────────────┘                           │
│                                 │                                        │
│         ┌───────────────────────┼───────────────────────┐              │
│         ▼                       ▼                         ▼             │
│  [Cashflow chart]       [Revenue vs OPEX]        [DTN load chart]      │
│  [Engineering log]      [KPI badges]             [QoS status badge]    │
└──────────────────────────────────────────────────────────────────────────┘

Technology stack: HTML5, Tailwind CSS (CDN), Chart.js, vanilla JavaScript — no build step required.

## DTN Contact Plan Model

### Network Topology

Ships (VHF/VDES uplink)  →  LEO Satellite  →  Ground Station  →  NOC/Cloud
       K ships/pass            N_sats              N_gs
       λ msgs/s/ship           h_orbit             downlink rate

The satellite acts as a store-and-forward relay: messages are buffered onboard during the uplink contact window, then dumped to ground stations during the downlink window.

### Contact Windows

| Link | Parameter | Default |
| --- | --- | --- |
| Uplink (ship → sat) | Rate C_up | 50 kbps |
| Uplink contact duration | Δ_up | 300 s |
| Downlink (sat → GS) | Rate C_down | 100 kbps |
| Downlink contact duration | Δ_down | 420 s |
| Orbit revisit (sat → GS) | T_rev | 90 min |

### Duty Cycles

δup=ΔupTrev,up,  δdown=ΔdownTrev,down

### End-to-End Capacity

Mup=Nsats⋅K⋅Cup⋅δupmbits/1000 [msgs/s]

Mdown=Nsats⋅Ngs⋅Cdown⋅δdownmbits/1000 [msgs/s]

Me2e=minMup, Mdown

The bottleneck is explicitly identified (Gargalo: UP or Gargalo: DOWN) in the Engineering Log.

### End-to-End Latency (no queue)

Davg=Trev,up2⏟wait for sat+Ttx,up+Trev,down2⏟wait for GS+Ttx,down [min]

## Business Model (ARPU)

### Default Service Stack

The ARPU is computed as a penetration-weighted sum of monthly per-ship service prices:

ARPUblended=i​pricei×peni100

| # | Service | Price ($/mo) | Penetration | Contribution |
| --- | --- | --- | --- | --- |
| 1 | AIS Tracking (base) | $15 | 100% | $15.00 |
| 2 | S-124 MSI — Safety | $50 | 85% | $42.50 |
| 3 | S-104 Weather / VTS | $120 | 60% | $72.00 |
| 4 | S-421 Route Plan | $80 | 50% | $40.00 |
| 5 | Fleet Analytics | $150 | 40% | $60.00 |
| 6 | Compliance / e-Nav | $60 | 70% | $42.00 |
|  | ARPU blended |  |  | ~$271.50/mo |

Design pattern: AIS Tracking at $15 (100%) is the acquisition anchor — it replicates the free-to-vessel AIS value proposition while providing a billing relationship. S-421 Route Planning and Fleet Analytics are the high-value upsell tier.

### Service tiers map to IMO e-Navigation S-mode standards

- S-124 — Navigational warnings (MSI)

- S-104 — Water level information (includes metocean/VTS gate passes)

- S-421 — Route plan exchange (vessel-to-vessel and vessel-to-VTS)

## Cost Structure

### CAPEX

| Item | Formula | Default |
| --- | --- | --- |
| Satellite unit cost | cost_sat × N_sats | $800k/sat |
| Ground station unit cost | cost_gs × N_gs | $500k/GS |
| Fixed initial CAPEX | Platform, licenses, integration | $1M |
| Total CAPEX₀ | N_sats×cost_sat + N_gs×cost_gs + fixed | ~$11.6M (12 sats, 4 GS) |

### OPEX (annual)

| Item | Formula | Default |
| --- | --- | --- |
| Satellite O&M | opex_sat × N_sats/year | $200k/sat/yr |
| Ground station O&M | opex_gs × N_gs/year | $50k/GS/yr |
| NOC + Staff (fixed) | annual overhead | $700k/yr |
| Variable OPEX | % × revenue (billing, S&M, support) | 10% of revenue |

## Financial Projections

### Customer Growth Model

Shipsyear=Shipsyear−1×1+g−c

where g = annual growth rate and c = annual churn rate.

If QoS enforcement is active, the billed ship count is capped to QoS ships max:

Shipsbilled=minShipsyear, Nshipsmax

### Annual Revenue

Revyear=Shipsbilled×ARPUblended×12

### Annual P&L

$$\text{OPEX}_{year} = N_{sats} \times \text{opex\\_sat} + N_{gs} \times \text{opex\\_gs} + \text{opex\\_fixed} + \text{opex\\_rev\\_pct} \times \text{Rev}_{year}$$

NetCFyear=Revyear−OPEXyear

CumulativeCFyear=−CAPEX0+t=1yearNetCFt

Payback year = first year where cumulative cashflow ≥ 0.

Horizon ROI:

ROI=CumulativeCFhorizonCAPEX0×100%

### Discounting and WACC

To account for the time value of money the simulator supports discounting of future cashflows using a Weighted Average Cost of Capital (WACC). Use the WACC slider in the UI to set the discount rate.

Per-year discounted cashflow (DCF):

DCFt=NetCFt1+WACCt

Net Present Value (NPV):

NPV=−CAPEX0+t=1TDCFt

Discounted payback year is the first year where the cumulative discounted cashflow (starting from -CAPEX₀) becomes ≥ 0:

CumDCFt=−CAPEX0+i=1tDCFi (find first t with CumDCFt≥0)

Discounted ROI (WACC adjusted):

ROIdisc=NPVCAPEX0×100%

If WACC = 0% the discounted metrics equal the nominal (undiscounted) metrics.

## KPIs & Outputs

| KPI | Location | Description |
| --- | --- | --- |
| Capacidade E2E | Top KPI bar | M_e2e in msgs/s |
| QoS ships max | Top KPI bar | Max ships at ρ_max utilization |
| Receita anual (Y1) | Top KPI bar | Year 1 gross revenue |
| Payback / ROI | Top KPI bar | Payback year + horizon ROI |
| ARPU blended | Services section | Weighted monthly ARPU |
| QoS Badge | Header | QoS OK (green) / QoS RISCO (red) |
| Engineering log | Log panel | Full parameter dump + diagnostics |

### Charts

| Chart | Type | Content |
| --- | --- | --- |
| Cashflow anual & acumulado | Bar + Line | Annual net cashflow (bars) + cumulative (line) |
| Receita vs OPEX | Stacked bar | Revenue vs total annual OPEX per year |
| DTN: carga vs capacidade | Line | Offered load vs M_e2e per year |

## Parameter Reference

### DTN / Contact Plan

| Input ID | Parameter | Range | Default |
| --- | --- | --- | --- |
| inp_nsats | Number of satellites | 1–48 | 12 |
| inp_ngs | Number of ground stations | 1–24 | 4 |
| inp_alt | Orbital altitude (km) | 400–1200 | 600 |
| inp_revisit | Sat→GS revisit period (min) | 10–180 | 90 |
| inp_k | Ships per satellite pass | 10–800 | 150 |
| inp_msg | Message size (bytes) | 256–16384 | 2048 |
| inp_lambda | Message rate (msgs/s/ship) | 0.001–0.2 | 0.010 |
| inp_qosutil | QoS utilization cap (%) | 50–95 | 85 |
| inp_cup | Uplink rate (kbps) | ≥1 | 50 |
| inp_dup | Uplink contact duration (s) | ≥1 | 300 |
| inp_cdown | Downlink rate (kbps) | ≥1 | 100 |
| inp_ddown | Downlink contact duration (s) | ≥1 | 420 |
| inp_enforce_qos | Enforce QoS cap on billed ships | checkbox | ✓ checked |

### Commercial Projection

| Input ID | Parameter | Range | Default |
| --- | --- | --- | --- |
| inp_baseShips | Ships in Year 1 | 100–20000 | 1750 |
| inp_growth | Annual growth rate (%) | −20–80 | 20 |
| inp_years | Projection horizon (years) | 3–20 | 10 |
| inp_churn | Annual churn rate (%) | 0–40 | 5 |
| inp_scale | Infrastructure scaling mode | see below | OFF |

### CAPEX / OPEX

| Input ID | Parameter | Default |
| --- | --- | --- |
| inp_cost_sat | Cost per satellite ($) ¦ 800,000 ¦ ¦ `inp_cost_gs` ¦ Cost per ground station ($) | 500,000 |
| inp_cost_fixed | Fixed initial CAPEX ($) ¦ 1,000,000 ¦ ¦ `inp_opex_sat_y` ¦ OPEX per satellite per year ($) | 200,000 |
| inp_opex_gs_y | OPEX per GS per year ($) ¦ 50,000 ¦ ¦ `inp_opex_fixed_y` ¦ Fixed annual OPEX — NOC/Staff ($) | 700,000 |
| inp_opex_rev_pct | Variable OPEX (% of revenue) | 10% |

## Default Scenario

With all parameters at default values:

Constellation:    12 satellites, 4 ground stations, h=600km
Contact plan:     C_up=50kbps/300s, C_down=100kbps/420s, T_rev=90min
Message profile:  K=150 ships/pass, λ=0.010 msg/s/ship, m=2048 bytes

M_e2e ≈ 2.5 msgs/s   (bottleneck: uplink)
QoS ships max ≈ 212 ships at ρ=85%

Year 1 fleet: 1750 ships → CAPPED to 212 ships (QoS RISCO)
Year 1 Revenue: 212 × $271.50 × 12 ≈ $690k
Year 1 OPEX:   12×$200k + 4×$50k + $700k + 10%×$690k ≈ $3.4M
Year 1 Net CF: ≈ −$2.7M

CAPEX₀ = 12×$800k + 4×$500k + $1M = $11.6M
Payback: not reached in 10-year horizon at 12 sats

Key insight: At 12 satellites, the QoS cap fundamentally limits revenue to ~212 ships. The constellation must scale to 36–48 satellites to address 600–850 ships and approach breakeven by Year 7–8.

## Key Formulas

### QoS Ships Maximum

Nshipsmax=ρmax×Me2eλ

### Offered Load

ρoffered=Nships×λMe2e

### Billed Ships (with QoS enforcement)

Nbilled=Nshipsif ρoffered≤ρmaxNshipsmaxotherwise

### Blended ARPU

ARPU=i=1npi×peni100 [$/month/ship]

## QoS Enforcement Logic

The inp_enforce_qos checkbox controls whether the billed fleet is capped to QoS ships max:

- Enabled (default): Ships beyond capacity are not billed — models the realistic scenario where the operator cannot sign up more customers than the network can serve. Revenue is conservative and accurate.

- Disabled: All contracted ships are billed regardless of network saturation — models an optimistic/oversubscription scenario. The Engineering Log will still report QoS RISCO.

The header QoS badge reflects current status:

| Badge | Condition |
| --- | --- |
| 🟢 QoS OK | Year 1 offered load ≤ ρ_max |
| 🔴 QoS RISCO | Year 1 offered load > ρ_max |

## Infrastructure Scaling Modes

The inp_scale dropdown controls automatic infrastructure growth when demand exceeds capacity:

| Mode | Behaviour |
| --- | --- |
| OFF (default) | Fixed constellation — CAPEX computed once at Year 0 |
| Auto-add GS | Ground stations are added as needed; satellites fixed |
| Auto-add satélites | Satellites are added as needed; GS fixed |
| Auto-add satélites e GS | Both GS and satellites added proportionally |

In scaling modes, new infrastructure CAPEX is added to the year in which it is deployed and reflected in the cumulative cashflow.

## Known Limitations

| # | Limitation | Impact |
| --- | --- | --- |
| 1 | No VDES session overhead (handshake + ARQ) modelled | Capacity is optimistic by ~15–30% |
| 2 | No satellite replacement/EOL cost (typical: 5-year nanosat lifetime) | CAPEX underestimated for 10-year horizon |
| 3 | Launch cost not included in CAPEX₀ | Missing ~$1–1.5M for rideshare (SpaceX Transporter) |
| 4 | No queuing delay model (M/D/1 or DTN bundle queue) | Latency estimates are lower bounds |
| 5 | No frequency management / AIS channel contention | VHF Ch87/88 collision not modelled |
| 6 | No NPV / discounted cashflow | ROI is undiscounted; add WACC ~8–12% for corrected IRR |
| 7 | Single ARPU for all ships | No geographic segmentation (SOLAS vs non-SOLAS) |

## Glossary

| Term | Definition |
| --- | --- |
| AIS | Automatic Identification System — IMO-mandated maritime transponder system (VHF Ch161.975/162.025 MHz) |
| VDES | VHF Data Exchange System — ITU-R M.2092 next-generation AIS successor with bidirectional data channels |
| WACC | Weighted Average Cost of Capital — the discount rate used to compute present value of future cashflows; expressed as a percent. Typical telecom/space WACC: 8–15%. |
| DCF | Discounted Cash Flow — a single year’s net cashflow discounted by (1+WACC)^t to reflect present value. |
| NPV | Net Present Value — sum of discounted cashflows minus initial CAPEX; positive NPV means value creation at the chosen WACC. |
| Discounted Payback | The first year where cumulative discounted cashflows (including -CAPEX at t=0) become non-negative. |
| DTN | Delay-Tolerant Networking — RFC 4838 architecture for intermittently connected networks; satellite LEO is a canonical DTN use case |
| Store-Carry-Forward | DTN routing paradigm: node stores bundle, carries it in orbit, forwards at next contact opportunity |
| Contact Plan | Scheduled set of communication opportunities between DTN nodes (ships ↔ sats ↔ GS) |
| ARPU | Average Revenue Per User (here: per vessel per month) |
| CAPEX | Capital Expenditure — one-time infrastructure investment |
| OPEX | Operational Expenditure — recurring annual costs |
| NOC | Network Operations Centre |
| S-124/S-104/S-421 | IHO/IMO S-mode data standards for MSI, water level information, and route plan exchange respectively |
| MSI | Maritime Safety Information — navigational warnings and meteorological alerts |
| QoS | Quality of Service — here expressed as maximum channel utilisation before congestion degrades delivery |
| λ (lambda) | Message arrival rate per ship (msgs/s) |
| ρ (rho) | Channel utilisation ratio (offered load / capacity) |
| M_e2e | End-to-end DTN capacity in messages per second |
| ROI | Return on Investment = (Cumulative Net CF at horizon) / CAPEX₀ |
| Payback | First year where Cumulative Cashflow ≥ 0 |
| Churn | Annual percentage of customers leaving the service |
| LEO | Low Earth Orbit — typically 400–1200 km altitude |

DTN VDES Business Simulator — v1.0 | February 2026